⚙️ Nonlinear Spring Equilibrium Using Bisection Method (MATLAB)

This project solves for the equilibrium displacement of a nonlinear spring subjected to a constant force using the Bisection method in MATLAB.

The spring contains a cubic stiffness component, making the system nonlinear. The equilibrium displacement is calculated iteratively by finding the root of the force residual equation.

📘 Problem Description

Given:

* Linear stiffness: k = 100

* Cubic stiffness: beta = 40

* Applied Force: P = 50

Using the balance of internal spring resistance and external applied force, we obtain the residual force equation:

This nonlinear equation is solved for x (displacement) when f(x) = 0 using the Bisection method.

🧠 Method Used

Bisection Update Equation:

Where:

* x_l = Lower bound (Initially 0)

* x_u = Upper bound (Initially 1.3)

* x_r = Midpoint (Current root estimate)

* y_l and y_r = Function evaluations at x_l and x_r

* Convergence condition: Loop continues while |x_u - x_l| > [span_13](start_span)0.01

The bounds are updated based on the sign of y_l \times y_r. If the product is negative, x_u = x_r; otherwise, x_l = x_r.

🛠 MATLAB Code

clc; clear; close all;

k = 100;    % Linear stiffness

beta = 40;  % Cubic stiffness

P = 50;     % Applied Force

xl = 0;

xu = 1.3;

err = 0.01;

iter = 1;

data = [];

while abs(xu - xl) > err

xr = (xl + xu) / 2;

yl = k*xl + beta*xl^3 - P;

yr = k*xr + beta*xr^3 - P;

data(iter, :) = [xl xr xu yl yr];

if yl * yr < 0

xu = xr;

else

xl = xr;

end

iter = iter + 1;

end

root = xr;

disp('   xl       xr       xu       yl       yr');

disp(data);

fprintf('Displacement Root = %f m\n', root);

x_vals = linspace(0, 1.3, 300);

y_vals = k*x_vals + beta*x_vals.^3 - P;

figure;

plot(x_vals, y_vals, 'LineWidth', 1.5); hold on;

plot(root, 0, 'ro', 'MarkerSize', 8, 'MarkerFaceColor', 'r');

grid on;

xlabel('Displacement (x)');

ylabel('Force Residual (f(x))');

title('Nonlinear Spring Equilibrium Root');

📊 Output

The program displays:

* A data table showing the tracking of variables (x_l, x_r, x_u, y_l, y_r) across all iterations.

* Final value of the calculated Displacement Root.

* A generated plot of the Force Residual vs. Displacement curve, with the exact equilibrium root marked with a red dot.

🚀 How to Run

* Open MATLAB

* Create a new script file (e.g., nonlinear_spring_bisection.m)

* Paste the code

* Run the script

📌 Applications

* Mechanical vibrations and spring analysis

* Numerical methods implementation

* Bisection method practice

* Structural engineering simulations